MnDOT 99007 – Evaluation of Guide Sign Fonts

02/28/2013

Project Status Update

Task 1: Kick-Off Meeting

Task 2: Topic Investigation

Task 3: Closed Course Testing

3A: Experimental Design

- The following three fonts will be tested:

  - Series E(Mod)

  - Enhanced E(Mod)

  - Clearview 5W

- Both letter and number legends will be tested as follows:

  - Letter legends will be tested using full-scale overhead guide signs.

  - Number legends will be tested using full-scale shoulder-mounted signs.

- High intensity sheeting will be used on all signs:

  - White letters (Type XI)

  - Green background (Type IV)

- Drivers in two age groups will be tested, with the focus of the study being on older drivers.

  - Younger drivers (21-35)

  - Older drivers (65+)

- Data will be collected for daytime and nighttime conditions, with the focus of the study being on nighttime driving and the halation effects of the high intensity sheeting.

3B: Equipment Setup

- The following full-scale roadway test signs (panels and overlays) have been ordered:

  - 2 overhead back panels (8 ft x 10 ft)

  - 54 letter legend overlays – 3 fonts, 18 words/font (2.5 ft x 8 ft)

  - 18 number legend panels – 3 fonts, 6 number combinations/font (2.5 ft x 4 ft)

- Forklifts will be used to support the overhead signs.

  - Forklift operator training will take place within the first two weeks of March.

- Experimental forms are being completed including:

  - Instructions for participants

  - Instructions for researchers

  - Data sheets

3C: Data Collection

- Portable GPS measurement systems will be used in vehicle(s)