TRANSPORTATION POOLED FUND PROGRAM

QUARTERLY PROGRESS REPORT

Lead Agency (FHWA or State DOT):  Washington State Department of Transportation

INSTRUCTIONS:

Project Managers and/or research project investigators should complete a quarterly progress report for each calendar quarter during which the projects are active.  Please provide a project schedule status of the research activities tied to each task that is defined in the proposal; a percentage completion of each task; a concise discussion (2 or 3 sentences) of the current status, including accomplishments and problems encountered, if any.  List all tasks, even if no work was done during this period.

| Transportation Pooled Fund Program Project # (i.e, SPR-2(XXX), SPR-3(XXX) or TPF-5(XXX) TPF-5(323) |  | Transportation Pooled Fund Program - Report Period: □Quarter 1 (January 1 – March 31) □Quarter 2 (April 1 – June 30) X□Quarter 3 (July 1 – September 30) □Quarter 4 (October 1 – December 31) |  |
| --- | --- | --- | --- |
| Project Title: Underwater Noise Attenuation Experimental Methods |  |  |  |
| Name of Project Manager(s): Jon Peterson | Phone Number: (360) 705-7499 |  | E-Mail peterjn@wsdot.wa.gov |
| Lead Agency Project ID: | Other Project ID (i.e., contract #): |  | Project Start Date: October 9, 2013 |
| Original Project End Date: December 2017 | Current Project End Date: |  | Number of Extensions: |

Project schedule status:

xx□ On schedule	□ On revised schedule		□ Ahead of schedule		□ Behind schedule

Overall Project Statistics:

| Total Project Budget | Total Cost to Date for Project | Percentage of Work Completed to Date |
| --- | --- | --- |
| $225,000 | 0 | 0 |

Quarterly Project Statistics:

| Total Project Expenses and Percentage This Quarter | Total Amount of Funds Expended This Quarter | Total Percentage of Time Used to Date |
| --- | --- | --- |

Project Description: Washington State Department of Transportation (WSDOT) regularly installs and replaces steel piles when maintaining and constructing ferry terminals and bridges. In 2002 while driving steel piles at a ferry terminal, pile perch were killed by the impact pile driving activities. Since then, NOAA Fisheries and US Fish and Wildlife Service (the Services) established thresholds for harm and harassment and developed minimization measures for pile driving projects designed to protect Endangered Species Act listed species from pile driving activities. Minimization measures include requiring the use of a bubble curtain sound attenuation device, limiting numbers of piles installed per day, requiring monitoring of the zones of harm and harassment and stopping pile driving if any listed birds or marine mammals enter the zones. All of these measures can increase the time it takes to construct a project. In 2004 WSDOT began conducting research on methods to reduce the effects of impact pile driving. Initial research focused on sound attenuation devices and a double walled, pile surrounding sleeve was tested. Underwater noise measurements for the purpose of evaluating the noise reduction performance of two new pile designs from the University of Washington were conducted on October 29-30, 2014 in Commencement Bay in cooperation with the Washington State Department of Transportation and Port of Tacoma.

Progress this Quarter (includes meetings, work plan status, contract status, significant progress, etc.): The researchers from the University of Washington and their partners in the testing are preparing the final report. WSDOT and its partners will be doing a full scale test of double wall piles in later in 2015 at the SR 160 Vashon Ferry Terminal – Timber Trestle and Terminal Replacement. The preliminary results show reduction in sound of 11 to 13 decibels for the double wall and mandrel test piles. The final report should be finished shortly. We received a copy of the draft Vashon Report (above) and gave the researchers substantial comments. The final report shu should be completed shortly. We held a conference call to discuss next steps and agreed to wait until results of the above report were released

Anticipated work next quarter: We agreed to wait on next steps until the following reports are reviewed: 1) Vashon Terminal Test Pile report will be final this quarter. 2) Will set up a conference call this quarter with the partners and Marine Construction Technologies (MCT) to discuss double wall piles and do the partners have any upcoming projects that can test the technology The partners will decide this quarter whether WSDOT plans to lead the technical committee and form a scope of work and prepare an RFP/RFQ to hire a researcher once the group decides on what they want tested. If the group can’t decide this quarter what to do, WSDOT will end the study and return the funds to the partner states.

Significant Results:

Circumstance affecting project or budget. (Please describe any challenges encountered or anticipated that might affect the completion of the project within the time, scope and fiscal constraints set forth in the agreement, along with recommended solutions to those problems).

Potential Implementation: