# Simplified CPT Performance-Based Assessment of Liquefaction and Effects:

# TASK 4

Prepared For:

Utah Department of Transportation

Research Division

Submitted By:

Brigham Young University

Department of Civil and Environmental Engineering

Authored By:

Kevin W. Franke

Tyler B. Coutu

Mikayla S. Hatch

Alex M. Arndt

Year 1, Quarter 3 Update Report for the

TPF-5(338) Technical Advisory Committee

April 2017

DISCLAIMER

The authors alone are responsible for the preparation and accuracy of the information, data, analysis, discussions, recommendations, and conclusions presented herein. The contents do not necessarily reflect the views, opinions, endorsements, or policies of the Utah Department of Transportation or the U.S. Department of Transportation. The Utah Department of Transportation makes no representation or warranty of any kind, and assumes no liability therefore.

#### ACKNOWLEDGMENTS

The authors acknowledge the Utah, Oregon, South Carolina, and Connecticut Departments of Transportation for funding this research for pooled fund study TPF-5(338). The views and opinions presented in this report represent those of its authors, and may not represent those of the state agencies funding this research.

TECHNICAL REPORT ABSTRACT

| 1. Report No. NA |  | 2. Government Accession No. NA |  |  | 3. Recipient's Catalog No. NA |  |
| --- | --- | --- | --- | --- | --- | --- |
| 4. Title and Subtitle Simplified CPT Performance-Based Assessment of Liquefaction and Effects: Task 4 |  |  |  |  | 5. Report Date April 2017 |  |
|  |  |  |  |  | 6. Performing Organization Code NA |  |
| 7. Author(s) Kevin W. Franke, Tyler B. Coutu, Mikayla S. Hatch, Alex M. Arndt |  |  |  |  | 8. Performing Organization Report No. NA |  |
| 9. Performing Organization Name and Address Brigham Young University Department of Civil and Environmental Engineering 368 Clyde Building Provo-UT 84602-4009 |  |  |  |  | 10. Work Unit No. 4207415D |  |
|  |  |  |  |  | 11. Contract or Grant No. 169826 |  |
| 12. Sponsoring Agency Name and Address Utah Department of Transportation 4501 South 2700 West P.O. Box 148410 Salt Lake City, UT 84114-8410 |  |  |  |  | 13. Type of Report & Period Covered Year 1, Quarter 3 TAC Update January – March 2017 |  |
|  |  |  |  |  | 14. Sponsoring Agency Code PIC No. UT15.402 |  |
| 15. Supplementary Notes Prepared in cooperation with the Utah Department of Transportation and the U.S. Department of Transportation, Federal Highway Administration |  |  |  |  |  |  |
| 16. Abstract The purpose of the research being performed is to provide the benefit of the full performance-based probabilistic earthquake hazard analysis, without requiring special software, training, and experience. To do this, simplified models of liquefaction triggering and lateral spread displacements that approximate the results of the full probabilistic analysis will be developed. The full projects scope of work will be completed in two-year segments. This report provides the year 1, quarter 3 progress update by addressing the completion of the Numerical tool, as well as an analysis of the results, for task 4 of the TPF-5(338) research contract. Year1, quarter 3 progress consists of finalization of a research tool to run the full performance-based probabilistic calculation of liquefaction triggering, liquefaction-induced settlement, and liquefaction-induced lateral displacement. This program, CPTLiquefY, currently has the capabilities of reading in a CPT file, running soil parameter calculations, pseudo-probabilistic liquefaction analyses, and deterministic liquefaction analyses. Full probabilistic methods are completely operational, along with a connection to a new off-line USGS deaggregation tool that gives access to the 2014 USGS deaggregation models. This program, and the corresponding results, have been analyzed over 10 different cities, and 20 CPT profiles. The results are compared to pseudo-probabilistic and semi-probabilistic procedures. |  |  |  |  |  |  |
| 17. Key Words Liquefaction Triggering, Performance-based Engineering, Seismic Hazards, Cone Penetration Test, USGS 2014 Deaggregation, Lateral Spread Displacement, Settlement, Probabilistic |  |  | 18. Distribution Statement Not restricted. Available through: UDOT Research Division 4501 South 2700 West P.O. Box 148410 Salt Lake City, UT 84114-8410 www.udot.utah.gov/go/research |  |  | 23. Registrant's Seal NA |
| 19. Security Classification (of this report) Unclassified | 20. Security Classification (of this page) Unclassified |  | 21. No. of Pages 104 | 22. Price NA |  |  |

TABLE OF CONTENTS

LIST OF FIGURES	6

UNIT CONVERSION FACTORS	13

LIST OF ACRONYMS	14

EXECUTIVE SUMMARY	16

1.0 INTRODUCTION	18

1.1 Problem Statement	18

1.2 Objectives	18

1.3 Scope	18

2.0 Changes Made and Completion of CPTLiquefY	19

2.1 Changes Made to CPTLiquefY	19

2.1.1 Inclusion of a Practical Limit for the Ku et al. Full-Probabilistic Method	19

2.1.2 Adjustment of Semi-Probabilistic Analysis	20

2.1.3 Lateral Spread Analysis Complete	20

2.1.4 Return Period is Displayed When Selected	21

2.1.5 USGS Uniform Hazard Tool Fully Integrated and Functional for West and East	21

2.2 Completion of CPTLiquefY	21

3.0 Methodology and Results	22

3.1 Methodology	22

3.1.1 Soil Profiles	22

3.1.2 Site Locations	24

3.1.3 Return Periods	26

3.2 Liquefaction Triggering Results	26

3.2.1 Example of Results from CPTLiquefY	26

3.2.2 Comparison of Ku et al. and Boulanger and Idriss Full-Probabilistic Results.	28

3.2.3 Variation of FSL between Sites	29

3.2.4 Variation of FSL between Return Periods	32

3.2.5 Variation of FS with Soil Profile Stiffness	33

3.2.6 Comparison of Full-Probabilistic Methods with Pseudo-Probabilistic Methods	35

3.2.7 Robertson and Wride Pseudo-probabilistic vs. Ku et al. Full-Probabilistic Method	36

3.2.8 Boulanger and Idriss Pseudo-probabilistic vs. Full-Probabilistic Methods	40

3.2.9 Summary of Comparisons	44

3.3 Settlement Results	50

3.3.1 Example of Results Calculated by CPTLiquefY	50

3.3.2 Comparison of Full-Probabilistic, Pseudo-Probabilistic, and Semi-Probabilistic	51

3.4 Lateral Spread Results	65

3.4.1 Example of Results Calculated by CPTLiquefY	66

3.4.2 Comparison of Full-Probabilistic, Pseudo-Probabilistic, and Semi-Probabilistic	66

4.0 SENSITIVITY ANALYSIS	81

4.1 Thin Layer Correction and Depth Weighting Factor	81

4.1.1 Depth Weighting Factor Correction	81

4.1.2 Thin Layer Correction	82

4.2 Sensitivity Analysis of Settlement	83

4.3 Sensitivity Analysis of Lateral Spread	86

5.0 CONCLUSIONS	91

5.1 Summary	91

5.2 Future Work	92

REFERENCES	93

6.0 APPENDIX	95

LIST OF FIGURES

Figure 2-1 Comparison of Ku et al. CRR values with and without a practical cap.	21

Figure 3-1 Stiffness of CPT profiles plotted at depth.	24

Figure 3-2 Map of all ten cities in this study.	26

Figure 3-3: Example Hazard curve output from CPTLiquefY.	28

Figure 3-4: Comparison of results from the full-probabilistic liquefaction analysis of profile 6 at Butte, MT relating  to depth in meters.	29

Figure 3-5: Comparison of  for both full-probabilistic methods (Return Period = 1039 years).	30

Figure 3-6:  results for profile 14 at a return period of 475 years.	31

Figure 3-7: results for profile 14 at a return period of 1039 years.	32

Figure 3-8: results for profile 14 at a return period of 2475 years.	32

Figure 3-9: Comparison of  for 3 return periods (profile 14, Salt Lake City).	33

Figure 3-10: Comparison of  results from different categories of profile stiffness.	35

Figure 3-11: Location of four quadrants on an example plot.	37

Figure 3-12: Comparison of  from Robertson and Wride conventional (Mean) and Ku et al. full-probabilistic approaches,  = 475 years.	38

Figure 3-13: Comparison of  from Robertson and Wride conventional (Modal) and Ku et al. full-probabilistic approaches,  = 475 years.	39

Figure 3-14: Comparison of  from Robertson and Wride conventional (Mean) and Ku et al. full-probabilistic approaches,  = 1039 years.	39

Figure 3-15: Comparison of  from Robertson and Wride conventional (Modal) and Ku et al. full-probabilistic approaches,  =1039 years.	40

Figure 3-16: Comparison of  from Robertson and Wride conventional (Mean) and Ku et al. full-probabilistic approaches,  = 2475 years.	40

Figure 3-17: Comparison of  from Robertson and Wride conventional (Modal) and Ku et al. full-probabilistic approaches,  = 2475 years.	41

Figure 3-18: Comparison of  for Boulanger and Idriss conventional (Mean) and full-probabilistic approaches,  = 475 years.	42

Figure 3-19: Comparison of  for Boulanger and Idriss conventional (Modal) and full-probabilistic approaches,  = 475 years.	43

Figure 3-20: Comparison of  for Boulanger and Idriss conventional (Mean) and full-probabilistic approaches,  = 1039 years.	43

Figure 3-21: Comparison of for Boulanger and Idriss conventional (Modal) and full-probabilistic approaches,  = 1039.	44

Figure 3-22: Comparison of for Boulanger and Idriss conventional (Mean) and full-probabilistic approaches,  = 2475.	44

Figure 3-23: Comparison of for Boulanger and Idriss conventional (Modal) and full-probabilistic approaches,  = 2475.	45

Figure 3-24: Probability of liquefaction values from study (using Ku et al. equation)  = 475.	49

Figure 3-25: Probability of liquefaction values from study (using Boulanger and Idriss equation)  = 475.	50

Figure 3-26: Example settlement hazard curve.	51

Figure 3-27: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 475 year return period using the Robertson and Wride liquefaction triggering procedure.	53

Figure 3-28: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 1039 year return period using the Robertson and Wride liquefaction triggering procedure.	54

Figure 3-29: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 2475 year return period using the Robertson and Wride liquefaction triggering procedure.	54

Figure 3-30: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 475 year return period using the Robertson and Wride liquefaction triggering procedure.	55

Figure 3-31: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 1039 year return period using the Robertson and Wride liquefaction triggering procedure.	55

Figure 3-32: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 2475 year return period using the Robertson and Wride liquefaction triggering procedure.	56

Figure 3-33: Semi-probabilistic versus fully-probabilistic for the 475 year return period using the Robertson and Wride liquefaction triggering procedure.	56

Figure 3-34: Semi-probabilistic versus fully-probabilistic for the 1039 year return period using the Robertson and Wride liquefaction triggering procedure.	57

Figure 3-35: Semi-probabilistic versus fully-probabilistic for the 2475 year return period using the Robertson and Wride liquefaction triggering procedure.	57

Figure 3-36: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 475 year return period using the Boulanger and Idriss liquefaction triggering procedure.	58

Figure 3-37: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 1039 year return period using the Boulanger and Idriss liquefaction triggering procedure.	58

Figure 3-38: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 2475 year return period using the Boulanger and Idriss liquefaction triggering procedure.	59

Figure 3-39: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 475 year return period using the Boulanger and Idriss liquefaction triggering procedure.	59

Figure 3-40: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 1039 year return period using the Boulanger and Idriss liquefaction triggering procedure.	60

Figure 3-41: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 2475 year return period using the Boulanger and Idriss liquefaction triggering procedure.	60

Figure 3-42: Semi-probabilistic versus fully-probabilistic for the 475 year return period using the Boulanger and Idriss liquefaction triggering procedure.	61

Figure 3-43: Semi-probabilistic versus fully-probabilistic for the 1039 year return period using the Boulanger and Idriss liquefaction triggering procedure.	61

Figure 3-44: Semi-probabilistic versus fully-probabilistic for the 2475 year return period using the Boulanger and Idriss liquefaction triggering procedure.	62

Figure 3-45: Box and whisker plots of actual return periods versus assumed 1039 year return period in the settlement analysis.	63

Figure 3-46: Box and whisker plots of actual return periods versus assumed 2475 year return period in the settlement analysis.	63

Figure 3-47: A heat map representing the number of CPT soundings, out of 20 soundings, in which the pseudo-probabilistic method under predicted settlement compared to the full-probabilistic procedure.	64

Figure 3-48 Example of lateral spread hazard curve.	67

Figure 3-49: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 475 year return period using the Robertson and Wride liquefaction triggering procedure.	69

Figure 3-50: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 1039 year return period using the Robertson and Wride liquefaction triggering procedure.	69

Figure 3-51: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 2475 year return period using the Robertson and Wride liquefaction triggering procedure.	70

Figure 3-52: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 475 year return period using the Robertson and Wride liquefaction triggering procedure.	70

Figure 3-53: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 1039 year return period using the Robertson and Wride liquefaction triggering procedure.	71

Figure 3-54: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 2475 year return period using the Robertson and Wride liquefaction triggering procedure.	71

Figure 3-55: Semi-probabilistic versus fully-probabilistic for the 475 year return period using the Robertson and Wride liquefaction triggering procedure.	72

Figure 3-56: Semi-probabilistic versus fully-probabilistic for the 1039 year return period using the Robertson and Wride liquefaction triggering procedure.	72

Figure 3-57: Semi-probabilistic versus fully-probabilistic for the 2475 year return period using the Robertson and Wride liquefaction triggering procedure.	73

Figure 3-58: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 475 year return period using the Boulanger and Idriss liquefaction triggering procedure.	73

Figure 3-59: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 1039 year return period using the Boulanger and Idriss liquefaction triggering procedure.	74

Figure 3-60: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 2475 year return period using the Boulanger and Idriss liquefaction triggering procedure.	74

Figure 3-61: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 475 year return period using the Boulanger and Idriss liquefaction triggering procedure.	75

Figure 3-62: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 1039 year return period using the Boulanger and Idriss liquefaction triggering procedure.	75

Figure 3-63: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 2475 year return period using the Boulanger and Idriss liquefaction triggering procedure.	76

Figure 3-64: Semi-probabilistic versus fully-probabilistic for the 475 year return period using the Boulanger and Idriss liquefaction triggering procedure.	76

Figure 3-65: Semi-probabilistic versus fully-probabilistic for the 1039 year return period using the Boulanger and Idriss liquefaction triggering procedure.	77

Figure 3-66: Semi-probabilistic versus fully-probabilistic for the 2475 year return period using the Boulanger and Idriss liquefaction triggering procedure.	77

Figure 3-67: Box and whisker plots of actual return periods versus	79

Figure 3-68: Box and whisker plots of actual return periods versus	79

Figure 4-1: Tip resistance analysis for thin sand layer (deposit A) interbedded within soft clay layer (deposit B). (Ahmadi & Robertson, 2005).	83

Figure 4-2: Box and whisker plots for R (settlement) at a return period of 475 years.	85

Figure 4-3: Box and whisker plots for R (settlement) at a return period of 1039 years.	86

Figure 4-4: Box and whisker plots for R (settlement) at a return period of 2475 years.	86

Figure 4-5: Box and whisker plots for R (lateral spread geometry 1) at a return period of 475 years.	87

Figure 4-6: Box and whisker plots for R (lateral spread geometry 1) at a return period of 1039 years.	88

Figure 4-7: Box and whisker plots for R (lateral spread geometry 1) at a return period of 2475 years.	88

Figure 4-8: Box and whisker plots for R (lateral spread geometry 3) at a return period of 475 years.	90

Figure 4-9: Box and whisker plots for R (lateral spread geometry 3) at a return period of 1039 years.	90

Figure 4-10: Box and whisker plots for R (lateral spread geometry 3) at a return period of 2475 years.	91

Figure 6-1: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 475 year return period using the Robertson and Wride liquefaction triggering procedure.	96

Figure 6-2: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 1039 year return period using the Robertson and Wride liquefaction triggering procedure.	97

Figure 6-3: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 2475 year return period using the Robertson and Wride liquefaction triggering procedure.	97

Figure 6-4: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 475 year return period using the Robertson and Wride liquefaction triggering procedure.	98

Figure 6-5: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 1039 year return period using the Robertson and Wride liquefaction triggering procedure.	98

Figure 6-6: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 2475 year return period using the Robertson and Wride liquefaction triggering procedure.	99

Figure 6-7: Semi-probabilistic versus fully-probabilistic for the 475 year return period using the Robertson and Wride liquefaction triggering procedure.	99

Figure 6-8: Semi-probabilistic versus fully-probabilistic for the 1039 year return period using the Robertson and Wride liquefaction triggering procedure.	100

Figure 6-9: Semi-probabilistic versus fully-probabilistic for the 2475 year return period using the Robertson and Wride liquefaction triggering procedure.	100

Figure 6-10: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 475 year return period using the Boulanger and Idriss liquefaction triggering procedure.	101

Figure 6-11: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 1039 year return period using the Boulanger and Idriss liquefaction triggering procedure.	101

Figure 6-12: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 2475 year return period using the Boulanger and Idriss liquefaction triggering procedure.	102

Figure 6-13: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 475 year return period using the Boulanger and Idriss liquefaction triggering procedure.	102

Figure 6-14: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 1039 year return period using the Boulanger and Idriss liquefaction triggering procedure.	103

Figure 6-15: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 2475 year return period using the Boulanger and Idriss liquefaction triggering procedure.	103

Figure 6-16: Semi-probabilistic versus fully-probabilistic for the 475 year return period using the Boulanger and Idriss liquefaction triggering procedure.	104

Figure 6-17: Semi-probabilistic versus fully-probabilistic for the 1039 year return period using the Boulanger and Idriss liquefaction triggering procedure.	104

Figure 6-18: Semi-probabilistic versus fully-probabilistic for the 2475 year return period using the Boulanger and Idriss liquefaction triggering procedure.	105

LIST OF TABLES

Table 3-1 Summary of Soil Profiles	24

Table 3-2 Selected Cities Used in Study	26

Table 3-3: Percentage Data in Each Quadrant: Robertson and Wride vs. Ku et al.  = 475 Years	46

Table 3-4: Percentage Data in Each Quadrant: Robertson and Wride vs. Ku et al.  = 1039     Years	46

Table 3-5: Percentage Data in Each Quadrant: Robertson and Wride vs. Ku et al.  = 2475     Years	47

Table 3-6: P Percentage Data in Each Quadrant: Boulanger and Idriss (Conventional) vs. Boulanger and Idriss (Full-Probabilistic)  = 475 Years	47

Table 3-7: Percentage Data in Each Quadrant: Boulanger and Idriss (Conventional) vs. Boulanger and Idriss (Full-Probabilistic)  = 1039 Years	47

Table 3-8: Percentage Data in Each Quadrant: Boulanger and Idriss (Conventional) vs. Boulanger and Idriss (Full-Probabilistic)  = 2475 Years	48

UNIT CONVERSION FACTORS

Units used in this report and not conforming to the UDOT standard unit of measurement (U.S. Customary system) are given below with their U.S. Customary equivalents:

LIST OF ACRONYMS

CPT		Cone Penetration Test

PBEE		Performance-Based Earthquake Engineering

SPT		Standard Penetration Test

TAC		Technical Advisory Committee

UDOT		Utah Department of Transportation

USGS		United States Geological Survey

LIST OF TERMS

CRR		cyclic resistance ratio

CSR		cyclic stress ratio

FS		Factor of Safety

DF		depth weighting factor

FSL		factor of safety against liquefaction triggering

H		free face height H

KH		depth correction factor

L		distance from toe of the free face

MSF		Magnitude Scaling Factor

PL		probability of liquefaction

qreq		cone tip resistance required to resist liquefaction

Qtncs		clean-sand equivalent normalized CPT tip resistance Robertson (2009)

R		ratio of baseline value to corrected value

S		ground slope in percentage

TR		return period

# EXECUTIVE SUMMARY

The purpose of this research is to provide the benefit of the full performance-based probabilistic liquefaction hazard analysis (PLHA), without requiring special software, training, and experience. To do this, full-probabilistic (performance-based) methods have been created to calculate liquefaction triggering, liquefaction-induced settlements, and liquefaction-induced lateral spread based on cone penetration test (CPT) data. A computer tool, CPTLiquefY, has been developed to perform the full-probabilistic hazard calculations. With the program completed, an analysis was done to compare the full-probabilistic approach to pseudo-probabilistic, and semi-probabilistic calculations. This report provides an update of the Quarter 3 Year 1 progress of development of the computer tool, addressing Task 4 of the TPF-5(338) research contract.

The main focus of Quarter 3 has been the completion of the tool to run site-specific full-probabilistic liquefaction analysis, CPTLiquefY. The other focus has been on the analysis of the comparison between different procedures previously mentioned. A few changes and adjustments were made to CPTLiquefY as the analysis began. This was done to correct slight errors, as well to make it more user-friendly. With the completed tool and data for liquefaction triggering, settlement, and lateral spread, the results could be analyzed for 20 CPT profiles and at 10 different locations.

The analysis of liquefaction triggering models appears to show that in general the new Boulanger and Idriss full-probabilistic procedure will give more conservative results than the Ku et al. full-probabilistic procedure. Other factors such as the seismicity of the location, stiffness of the soil profile, and return period of interest are seen to have a significant effect on the results of these procedures.

The results from the comparative study indicated similar trends between the liquefaction-induced settlements and lateral spreading. The data suggests for a low return period, the pseudo-probabilistic settlement and lateral spread values correlated fairly well with full-probabilistic values. However, at medium to high return periods, this correlation deteriorated and showed pseudo-probabilistic methods under-predicting settlements and lateral spreading significantly. The semi-probabilistic method followed similar trends to the pseudo-probabilistic method. These trends mirror the trends found in the SPT study and indicate current practices are significantly under-predicting seismic hazards.

A sensitivity analysis was completed which compares the full-probabilistic results of settlement and lateral spread to those when a thin layer correction and a depth factor correction is applied. It was found that settlement and lateral spread displacements both decrease significantly when a depth factor is applied, and only slightly when the thin layer correction is applied. These results are consistent across multiple return periods.

With all data collected and analyzed, Task 4 of the TPF-5(338) research contract has been completed. The next step is to begin the second phase of this research project: development of simplified procedures and reference parameter maps

# INTRODUCTION

## Problem Statement

The purpose of Task 4 of this project is to develop software that calculates a full-probabilistic engineering analysis to predict liquefaction triggering, lateral spread, and post-liquefaction free-field settlement. To accomplish this purpose, empirical and/or semi-empirical models are used to estimate soil parameters from a cone penetration test, CPT, as well as determine the behavior of the given soil during and after a seismic event. This is all accomplished using code written in Visual C++ to automate these procedures.

## Objectives

The objective of this report is to report on the completion of the software CPTLiquefY, as well as the results received, addressing Task 4 of the TPF-5(338) research contract.

## Scope

The primary task of this phase of research is the completion of software that calculates soil characteristics from a CPT and automates the full-probabilistic model for liquefaction triggering, lateral spread, and post-liquefaction settlement.

This report is organized to include the following sections:

- Introduction

- Changes Made to CPTLiquefY Since Last Report and TAC Presentation

- Methodology for Running Analyses

- Results of CPTLiquefY for Liquefaction Triggering

- Results of CPTLiquefY for Settlement

- Results of CPTLiquefY for Lateral Spread

- Sensitivity Analysis of Thin Layer Correction and Depth Weighting Factor

- Conclusions

- Appendices

# Changes Made and Completion of CPTLiquefY

## Changes Made to CPTLiquefY

Since January 2017, some changes and slight adjustments have been made to CPTLiquefY. These changes include and adjustment of the liquefaction triggering calculation, semi-probabilistic analysis, completion of the lateral spread analysis, adjusted display of selected return period, and full integration of the United States Geologic Survey (USGS) Uniform Hazard Tool.

### Inclusion of a Practical Limit for the Ku et al. Full-Probabilistic Method

In attempting to implement the Ku et al. (probabilistic version of the Robertson and Wride) method into a full-probabilistic setup, certain limitations to the model were detected. Because in a full-probabilistic method even extremely rare circumstances are considered, problems arose with the model at very high levels of seismic loading. In the cases of extremely high seismic loading (CSR=1.5 or greater) the Ku et al. method gives unrealistically high values for the CPT resistance required to resist liquefaction (). This problem was found to arise from the deterministic CRR equations on which the Ku et al. models is based. The limit to where these equations fit the case history data is about Qtncs = 165.

In order to solve this problem, an equation was created to represent the cyclic resistance for values beyond Qtncs = 165. This equation is based on the CRR equation used in the Boulanger and Idriss (2014) method which more accurately represents the cyclic resistance of a soil at very high values of seismic loading. The results of this change can be viewed in Figure 2-1; where the solid line represents the CRR curve with the practical limit, and the dotted line shows the CRR curve using the original CRR equations beyond their original bounds (Qtncs = 165). These CRR curves essentially represent the boundary between cases which are expected to liquefy (to the left) and those which are not expected to liquefy (to the right of the curve). The area between the two lines represents the location that unrealistic  values would be calculated in the full-probabilistic analysis without the use of a practical limit.

Figure 2-1 Comparison of Ku et al. CRR values with and without a practical cap.

### Adjustment of Semi-Probabilistic Analysis

CPTLiquefY has the ability to run deterministic, pseudo-probabilistic, full-probabilistic and semi-probabilistic analyses. The semi-probabilistic analysis takes into account the uncertainty when determining the liquefaction triggering. However, it does not account for uncertainty in the settlement or lateral spread calculations. It was discovered that this analysis was being run incorrectly in CPTLiquefY. The settlement and lateral spread calculations were not using the corresponding return period in the completed liquefaction triggering analysis. After adjusting for this slight error, the results are more in line with what was expected and what was observed in the SPT study. See the following sections for the presentation of these results.

### Lateral Spread Analysis Complete

The lateral spread analysis was completed for all different analysis methods. The lateral spread calculations are similar to those found in the settlement procedure, however the conversion from horizontal strain to lateral spread are slightly more complex than the conversion from vertical strain to settlement. This calculation was modeled after the Zhang et al (2004) procedure.

### Return Period is Displayed When Selected

During the Technical Advisory Committee (TAC) meeting on February 22, 2017, it was suggested that an adjustment be made to the display of the selected return period for the pseudo-probabilistic analysis. At the time, the return period wasn’t displayed until after the analysis was complete. Since then, immediately after selecting the mean annual rate of exceedance, the return period is displayed. This allows the user to ensure that the correct return period is selected before running the analysis.

### USGS Uniform Hazard Tool Fully Integrated and Functional for West and East

As USGS transitioned from their online database to the new Uniform Hazard Tool there has been some difficulty integrating it into CPTLiquefY. Previously the tool was only able to accurately retrieve data for the western states (not any central or eastern states). After working with USGS, the errors in their tool affecting the central and eastern states are corrected, and the deaggregation now runs seamlessly for any location.

## Completion of CPTLiquefY

After making adjustments and corrections (both those mentioned and other small refinements) CPTLiquefY is now complete. With the ability to run multiple locations and soil profiles at once, the analysis process takes some time, but is not unreasonable (approximately 12 minutes to run a full-probabilistic analysis at one location for one soil profile). Also with the ability to export results to Microsoft Excel, plots can be made to view hazard curves or to compare the results between difference methods. The following section discusses the results obtained, as well as the methodology behind the chosen parameters.

# Methodology and Results

## Methodology

To perform a thorough comparison of design methods, a comprehensive range of various soil conditions and site seismicity levels needed to be considered. The methods used to create a thorough comparative study are discussed below.

### Soil Profiles

Twenty actual CPT soundings are selected, containing a comprehensive range of soil stiffness and type. These CPT soundings are collected from the United States Geologic Survey (USGS) database of CPT data (https://earthquake.usgs.gov/research/cpt/data/table/). The CPT profiles are selected with the intention to have a thorough range of soil type and relative density/stiffness. This range in relative density/stiffness is evident by investigating the plot of corrected cone tip resistance (Qtncs) shown in Figure 3-1. Note the lack of holes” (i.e., white space) in Figure 3-1.

Figure 3-1 shows how the chosen profiles adequately cover all potentially impactful Qtncs, or stiffness, values across the depth. Only a few profiles are found with a Qtncs value less than 50kPa, because there are very few soils that exist naturally that are soft enough to have such a low value of Qtncs.  Also, any soil with a Qtncs value greater than 250kPa is automatically considered too dense to liquefy per the liquefaction triggering databases of Boulanger and Idriss (2014) and Robertson and Wride (1998). Therefore, it is not imperative to collect comprehensive Qtncs data greater than 250kPa.  All collected profiles are summarized in Table 3-1.

Figure 3-1 Stiffness of CPT profiles plotted at depth.

Table 3-1 Summary of Soil Profiles

To accurately demonstrate the differences between design methods, variables such as total depth and water table depth were standardized. For this study, all profiles were limited to 12 meters regardless of the full profile depth available from the database. This depth was chosen because after about 12 m soil layers will have little to no effect on ground surface settlements r lateral spreading, and in part because 12m has been used in other performance-based (full-probabilistic) studies (Franke et al. 2014). The depth of the water table can have a considerable impact on the amout of liquefiable soil layers. For liquefaction to occur soils have to be saturated, therefore any soils above the water table cannot liquefy. For this study, the water table was assumed to be at the ground surface. Finally, none of the CPT profiles from the USGS database contained a cone pore water pressure reading. For this study, the cone pore water pressure was assumed to be zero for each soil layer.

Although altering the water table depth and assuming zero for the cone pore water pressure will produce inaccurate estimations of site-specific liquefaction hazard effects, the purpose of this study is to compare the calculations from different design methods. These changes would be problematic if the purpose was to accurately predict liquefaction effects at a specific site; but because the purpose of this study is to compare calculation methods, these alterations do not affect the accuracy of this study’s results.

In order to perform an analysis of lateral spread, the site geometry had to be specified. Two different geometries were used to ensure consistency of results. The first geometry was for a gently sloping ground with the ground slope, S, of 3%. The second geometry was a level ground with a free face, where the free face height, H, is 6m, and the length of the level ground, L, is 50m. These geometries were chosen as they have been used in previous studies.

### Site Locations

To address the potential variable levels of seismicity a site could have, ten different cities are examined in this study. Each city is chosen to represent a different level of seismicity and in part because they have been used in other PBEE studies (Kramer and Mayfield 2007, Franke et al. 2014).  The chosen cities are distributed as such: 4 on the west coast near the San Andreas Fault, 2 in the pacific north-west near the Cascadia Subduction zone and associated faults, 2 near the Wasatch fault and rocky mountain region, 1 near the New Madrid Seismic Zone, and 1 near the Charleston Seismic Zone.  A map of the cities are shown in Figure 3-2 and a list of the cities with their corresponding latitude, longitude, mean/modal magnitudes is presented in Table 3-2.

Figure 3-2 Map of all ten cities in this study.

Table 3-2 Selected Cities Used in Study

### Return Periods

Every structure is designed for a different level of earthquake depending on either the importance of that structure (e.g., hospitals, fire stations, etc.) or the level of negative impact resulting from structural failure (e.g., nuclear facilities, football stadiums, etc.). Less critical structures are designed for seismic events with shorter return periods, in other words smaller seismic events, while more critical structures are designed for higher return periods. This study focuses on return periods 475 years, 1039 years, and 2475 years, which correspond to probabilities of exceedance of 15% in 75 years, 7% in 75 years, and 3% in 75 years, respectively. These return periods represent relatively low, medium, and high levels of seismic loading, respectively.

## Liquefaction Triggering Results

At the completion of the liquefaction analysis, all liquefaction triggering results were compiled into one database. Comparisons were made between the results of both of the new full-probabilistic models based on Ku et al. (2012) and Boulanger and Idriss (2014). A comparison was also made between the full-probabilistic results and results from more conventional methods of liquefaction analysis, such as the Robertson and Wride (1998) and Boulanger and Idriss (2014) pseudo-probabilistic methods. General trends associated with these comparisons are presented.

### Example of Results from CPTLiquefY

After completion of full-probabilistic calculations in CPTLiquefY, the liquefaction triggering results can be viewed on the “Triggering Results” tab. The data provided on this tab represents hazard curves for each individual soil layer analyzed in the input soil profile. When the results are plotted, a hazard curve like those shown in Figure 3-3 is produced. This curve relates design values for the factor of safety against liquefaction () with corresponding rates of exceedance (inverse of return period). The curves in Figure 3-3 show the results from both the Ku et al. and Boulanger and Idriss full-probabilistic methods along with a horizontal line that represents the mean annual rate of exceedance that correlates to a 475 year return period. In order to obtain design values from this curve, values should be interpolated from the hazard curve for a return period of interest. For example, the Boulanger and Idriss method has a value of about  at the 475 year return period, while the Ku et al. method gives a value of. Given that these values are less than, both methods would in this case predict the initiation of liquefaction for the considered soil layer.

Figure 3-3: Example Hazard curve output from CPTLiquefY.

In order to consider the liquefaction triggering hazard for a full soil profile using the full-probabilistic results, values must be chosen from each layer in the soil profile at a selected return period. CPTLiquefY has the capability to select these values and then export a clean excel file with the results from the analyzed soil profile. The results are then easily plotted as a profile. Figure 3-4 presents an example of a completed soil profile comparing  with depth. With this profile it is possible to analyze the liquefaction hazard at any depth, as well as compare the results from the different methods and results from different return periods of interest.

Figure 3-4: Comparison of results from the full-probabilistic liquefaction analysis of profile 6 at Butte, MT relating  to depth in meters.

### Comparison of Ku et al. and Boulanger and Idriss Full-Probabilistic Results.

Using the results from the analysis conducted with 20 soil profiles and 10 locations, a dataset of full-probabilistic results was formed for both methods. The comparison of these results with the Ku et al. full-probabilistic results on the x-axis and Boulanger and Idriss full-probabilistic results on the y-axis can be seen in Figure 3-5. From the plot, the following trends were noted. First, the Ku et al. methods tend to predict higher values for  if the computed  is less than one. For values of  greater than one, it becomes much more likely that the computed from each method could be similar, or the Boulanger and Idriss could predict larger values. Second, data scatter seems to increase when the FS approaches the range of one-to-one.4. In this region the calculation of FS is highly varied depending on other factors than the full-probabilistic model (profile stiffness, location). It can be noted that a large number of points can be seen on the = 2 line on each axis. This is because a limit was placed at a value of  2, which is a design value at which liquefaction will generally not be a concern. Without the application of this limit, the trends seen in Figure 3-5 simply continue beyond the shown values.

FSL for Full-Probabilistic Ku et al.

FSL for Full-Probabilistic Boulanger and Idriss

Figure 3-5: Comparison of  for both full-probabilistic methods (Return Period = 1039 years).

### Variation of FSL between Sites

To see the effect of individual factors on the computed  values, the results from the previous analysis were limited to one profile. To obtain a wide range of  values, a moderately liquefiable soil profile (profile 14) was chosen as an example. The results shown in Figure 3-6 illustrate the effect of location on. The lowest seismicity site (Butte) has the most values above = 1 and the highest seismicity site (Eureka) has the most below = 1. Interestingly, the two methods on average seem to predict more consistent values at low seismicity locations. Similar behaviors are also observed at return periods of 1033 years (Figure 3-7) and 2475 years (Figure 3-8).

FSL for Full-Probabilistic Ku et al.

FSL for Full-Probabilistic Boulanger and Idriss

Figure 3-6:  results for profile 14 at a return period of 475 years.

FSL for Full-Probabilistic Ku et al.

FSL for Full-Probabilistic Boulanger and Idriss

Figure 3-7: results for profile 14 at a return period of 1039 years.

FSL for Full-Probabilistic Ku et al.

FSL for Full-Probabilistic Boulanger and Idriss

Figure 3-8: results for profile 14 at a return period of 2475 years.

### Variation of FSL between Return Periods

As suggested previously, the return period of the analysis appears to have a significant effect on the comparison of the two full-probabilistic methods. To more clearly show this effect, a plot of one soil profile at one location of moderate seismicity is shown in Figure 3-9. As the return period increases, the values drift farther from the one-to-one line which represents where the two methods would calculate identical results. This can be explained by the tendency of the Boulanger and Idriss method to predict significantly smaller values for  if  is less than 1, while predicting similar or even greater values if  is greater than 1.

FSL for Full-Probabilistic Ku et al.

FSL for Full-Probabilistic Boulanger and Idriss

Figure 3-9: Comparison of  for 3 return periods (profile 14, Salt Lake City).

### Variation of FS with Soil Profile Stiffness

Before beginning the analysis, all 20 soil profiles were divided into three basic levels of stiffness; soft or very soft, moderate, and stiff. Figure 3-10 shows the general trends caused by the profile stiffness. Predictably, soft profiles tended to have lower values of, as well as the greatest discrepancy between the two methods. Stiff profiles, on the other hand generally had higher values of, but maintained the same trends between methods at low values of. Values above  > 1 tended to give larger  values for the Boulanger and Idriss method. Finally, moderately stiff profiles fit well between the soft, and stiff profiles, and showed a good range of behavior seen in both the soft, and stiff profiles. This likely takes place because many of the moderate profiles contain a mix of different soft and stiff layers along with layers with actual stiffness values that would be considered moderately stiff.

FSL for Full-Probabilistic Ku et al.

FSL for Full-Probabilistic Boulanger and Idriss

FSL for Full-Probabilistic Ku et al.

FSL for Full-Probabilistic Boulanger and Idriss

FSL for Full-Probabilistic Ku et al.

FSL for Full-Probabilistic Boulanger and Idriss

Figure 3-10: Comparison of  results from different categories of profile stiffness.

### Comparison of Full-Probabilistic Methods with Pseudo-Probabilistic Methods

A comparison of the full-probabilistic results with the results from the conventional method is helpful in comparing the difference between engineering design values currently calculated by practicing engineers, and engineering design values calculated from a performance-based (full-probabilistic) design method. The pseudo-probabilistic results in this section were computed for each soil sublayer in the 20 soil profiles from a PSHA using either mean or modal values. All 20 CPT profiles and all 10 sites discussed previously were included in the body of the data to show trends that correlate to a wide range of conditions.

The following results are displayed in a scatter plot format with conventional values for all of the soil sublayers on the y-axis and fully-probabilistic values for all of the soil sublayers on the x-axis. If the two methods were to compute identical values, the data points would fall directly on the 1:1 line (blue) displayed in the following figures. Lines at the value which divide predicted liquefied and non-liquefied behavior () (red) are drawn from each axis on the plot to divide the plot into 4 quadrants (Figure 3-11). The four quadrants are defined as:

- Top Left- The full-probabilistic method predicts liquefaction, the pseudo-probabilistic method does not

- Top Right- The full-probabilistic method and pseudo-probabilistic method both predict no liquefaction.

- Bottom Left- The full-probabilistic method and pseudo-probabilistic method both predict liquefaction.

- Bottom Right- The full-probabilistic method predicts no liquefaction, while the pseudo-probabilistic method predicts liquefaction.

FSL for Full-Probabilistic Ku et al.

FSL for Pseudo-Probabilistic Robertson and Wride (Mean)

Figure 3-11: Location of four quadrants on an example plot.

### Robertson and Wride Pseudo-probabilistic vs. Ku et al. Full-Probabilistic Method

The results of the comparison between the Robertson and Wride pseudo-probabilistic and the Ku et al. full-probabilistic can be seen from Figure 3-12 to Figure 3-17. These figures illustrate that for  values less than 1, the conventional method generally predicts smaller values of  when compared to the full-probabilistic method. However, it appears that most of the plotted points from the full comparison lie in quadrants 2 and 3, indicating a common prediction on whether liquefaction is expected to occur. The statistical distribution of the points will be discussed later in this chapter.

These plots also appear to show that for values of  greater than 1, the conventional method will generally give higher values for  than the full-probabilistic method. This could be significant if a different value of is used as the boundary between liquefied and non-liquefied behavior (ex. =1.2). In that case, the distribution of data in each quadrant would change.  Differences between the mean and modal methods appears to be limited to small shifts of the trends to the left or right caused by values obtained from the PSHA (low seismicity shifts left for modal, high seismicity shift right for modal.) From observation of the figures, it also appears that the data tends to aggregate in bands based on the seismic loading at the different locations used in the analysis.

FSL for Full-Probabilistic Ku et al.

FSL for Pseudo-Probabilistic Robertson and Wride (Mean)

Figure 3-12: Comparison of  from Robertson and Wride conventional (Mean) and Ku et al. full-probabilistic approaches,  = 475 years.

FSL for Full-Probabilistic Ku et al.

FSL for Pseudo-Probabilistic Robertson and Wride (Modal)

Figure 3-13: Comparison of  from Robertson and Wride conventional (Modal) and Ku et al. full-probabilistic approaches,  = 475 years.

FSL for Full-Probabilistic Ku et al.

FSL for Pseudo-Probabilistic Robertson and Wride (Mean)

Figure 3-14: Comparison of  from Robertson and Wride conventional (Mean) and Ku et al. full-probabilistic approaches,  = 1039 years.

FSL for Full-Probabilistic Ku et al.

FSL for Pseudo-Probabilistic Robertson and Wride (Modal)

Figure 3-15: Comparison of  from Robertson and Wride conventional (Modal) and Ku et al. full-probabilistic approaches,  =1039 years.

FSL for Full-Probabilistic Ku et al.

FSL for Pseudo-Probabilistic Robertson and Wride (Mean)

Figure 3-16: Comparison of  from Robertson and Wride conventional (Mean) and Ku et al. full-probabilistic approaches,  = 2475 years.

FSL for Full-Probabilistic Ku et al.

FSL for Pseudo-Probabilistic Robertson and Wride (Modal)

Figure 3-17: Comparison of  from Robertson and Wride conventional (Modal) and Ku et al. full-probabilistic approaches,  = 2475 years.

### Boulanger and Idriss Pseudo-probabilistic vs. Full-Probabilistic Methods

The trends noted in the comparison of the Boulanger and Idriss methods are somewhat different than those from the previous methods. While the previous comparisons showed a curved relationship, the results in Figure 3-18 to Figure 3-23 show a more linear relationship. Because of the general linearity of the relationship, the results from the conventional analysis will generally either predict higher values for  for most possible values of, or predict lower values of  for all possible values of. Although few of the predictions line up with the 1:1 line, relatively few data points seem to appear in quadrants 1 or 4, which represent a disagreement on the prediction of liquefaction triggering between the methods.

Similar to the previous comparison, the results are seen to be separated into bands relating to the location of each analysis. An interesting observation from this comparison is that data scatter seems to increase when using modal values instead of mean values for the pseudo-probabilistic analysis. Uniquely, this comparison showed divergent behavior, in that the comparison between the conventional and full-probabilistic methods are very close at very low values of , but the predictions are farther apart at values of higher. Of particular interest are values close to =1. It is at =1 that the conventional and full-probabilistic methods would disagree in this analysis. It can be seen that at =1 there is a significant difference between the methods, unlike at a lower value like =.2, where the methods are in almost total agreement. This trend signifies that there should be some degree of disagreement between the methods on the prediction of liquefaction initiation.

FSL for Full-Probabilistic Boulanger and Idriss

FSL for Pseudo-Probabilistic Boulanger and Idriss (Mean)

Figure 3-18: Comparison of  for Boulanger and Idriss conventional (Mean) and full-probabilistic approaches,  = 475 years.

FSL for Full-Probabilistic Boulanger and Idriss

FSL for Pseudo-Probabilistic Boulanger and Idriss (Modal)

Figure 3-19: Comparison of  for Boulanger and Idriss conventional (Modal) and full-probabilistic approaches,  = 475 years.

FSL for Full-Probabilistic Boulanger and Idriss

FSL for Pseudo-Probabilistic Boulanger and Idriss (Mean)

Figure 3-20: Comparison of  for Boulanger and Idriss conventional (Mean) and full-probabilistic approaches,  = 1039 years.

FSL for Full-Probabilistic Boulanger and Idriss

FSL for Pseudo-Probabilistic Boulanger and Idriss (Modal)

Figure 3-21: Comparison of for Boulanger and Idriss conventional (Modal) and full-probabilistic approaches,  = 1039.

FSL for Full-Probabilistic Boulanger and Idriss

FSL for Pseudo-Probabilistic Boulanger and Idriss (Mean)

Figure 3-22: Comparison of for Boulanger and Idriss conventional (Mean) and full-probabilistic approaches,  = 2475.

FSL for Full-Probabilistic Boulanger and Idriss

FSL for Pseudo-Probabilistic Boulanger and Idriss (Modal)

Figure 3-23: Comparison of for Boulanger and Idriss conventional (Modal) and full-probabilistic approaches,  = 2475.

### Summary of Comparisons

A brief statistical analysis of the distribution of the liquefaction triggering data in each of the four defined quadrants was conducted. A total of 45,590 different liquefaction triggering predictions were analyzed from each of the plots in the previous sections. The results of this analysis can be viewed in Table 3-3 through Table 3-8.

These results suggest that the full-probabilistic and conventional liquefaction triggering analysis methods agree on the prediction of liquefaction triggering around 95% to 99% of the time. The comparison of the Robertson and Wride pseudo-probabilistic vs. the Ku et al. full-probabilistic method showed that about 2 to 4 % of the time the performance-based method predicted the non-occurrence of liquefaction, while the pseudo-probabilistic predicted liquefaction to occur. Conversely, generally less than 1% of the cases represented predictions of non-liquefaction by the pseudo-probabilistic method and liquefaction by the full-probabilistic method. These values were fairly constant across the different return periods analyzed as well as when mean or modal values were used in the pseudo-probabilistic analysis.

The Boulanger and Idriss comparison values had a smaller percentage of points in quadrants 1 and 4 (different liquefaction triggering predictions). At low return periods, the full-probabilistic method predicted non-liquefaction while the conventional method predicted liquefaction about 1.5 percent of the time. This value drops to well below 1 percent for higher return periods. Trends for the opposite prediction (pseudo-probabilistic predicts non-liquefaction and full-probabilistic predicting liquefaction), followed a similar trend but in the opposite direction (percentage increase from low to high return periods).

Table 3-3: Percentage Data in Each Quadrant: Robertson and Wride vs. Ku et al.  = 475 Years

| Robertson and Wride vs. Ku et al. |  |  |  |
| --- | --- | --- | --- |
| 475 |  |  |  |
| Mean |  | Modal |  |
| 0.54% | 71.08% | 1.02% | 69.52% |
| 26.02% | 2.36% | 25.54% | 3.92% |

Table 3-4: Percentage Data in Each Quadrant: Robertson and Wride vs. Ku et al.  = 1039 Years

| Robertson and Wride vs. Ku et al. |  |  |  |
| --- | --- | --- | --- |
| 1039 |  |  |  |
| Mean |  | Modal |  |
| 0.92% | 63.88% | 1.21% | 63.25% |
| 33.17% | 2.03% | 32.88% | 2.65% |

Table 3-5: Percentage Data in Each Quadrant: Robertson and Wride vs. Ku et al.  = 2475 Years

| Robertson and Wride vs. Ku et al. |  |  |  |
| --- | --- | --- | --- |
| 2475 |  |  |  |
| Mean |  | Modal |  |
| 0.79% | 59.17% | 1.33% | 58.61% |
| 36.67% | 3.37% | 36.13% | 3.93% |

Table 3-6: P Percentage Data in Each Quadrant: Boulanger and Idriss (Conventional) vs. Boulanger and Idriss (Full-Probabilistic)  = 475 Years

| B&I pseudo-probabilistic vs. B&I full-probabilistic |  |  |  |
| --- | --- | --- | --- |
| 475 |  |  |  |
| Mean |  | Modal |  |
| 0.35% | 63.16% | 0.79% | 62.99% |
| 35.12% | 1.38% | 34.68% | 1.55% |

Table 3-7: Percentage Data in Each Quadrant: Boulanger and Idriss (Conventional) vs. Boulanger and Idriss (Full-Probabilistic)  = 1039 Years

| B&I pseudo-probabilistic vs. B&I full-probabilistic |  |  |  |
| --- | --- | --- | --- |
| 1039 |  |  |  |
| Mean |  | Modal |  |
| 0.83% | 59.57% | 1.04% | 59.37% |
| 39.42% | 0.18% | 39.21% | 0.38% |

Table 3-8: Percentage Data in Each Quadrant: Boulanger and Idriss (Conventional) vs. Boulanger and Idriss (Full-Probabilistic)  = 2475 Years

| B&I pseudo-probabilistic vs. B&I full-probabilistic |  |  |  |
| --- | --- | --- | --- |
| 2475 |  |  |  |
| Mean |  | Modal |  |
| 1.03% | 57.39% | 1.28% | 57.22% |
| 41.52% | 0.05% | 41.28% | 0.22% |

The significance of these results is that potentially, in 1 to 4 percent of cases, the conventional liquefaction triggering methods are clearly over predicting the initiation of liquefaction. Although this would make it appear that in 95% or greater of cases that the prediction of liquefaction hazard would be the same with both the pseudo-probabilistic and performance-based methods, this is not necessarily the case. Because of the trends noted in the figures above, on average, it appears that the performance-based method will still give lower values of  than the pseudo-probabilistic method. To explain how the methods do not necessarily compute equivalent values for liquefaction hazard, an example is provided.

Consider the hypothetical case of a prediction of = 0.9 from the full-probabilistic method, and a prediction of = 0.3 from the pseudo-probabilistic method. In this case, both predictions would indicate liquefaction, and would thus plot in quadrant 4. Although these values both represent the same liquefaction initiation prediction, they do not represent the same liquefaction initiation hazard. In terms of, it is somewhat difficult to compare the actual liquefaction initiation hazard of these predictions. In order to compare the liquefaction triggering hazard in a more objective manner,  can be used instead of . If doing so, a  value of 0.9 results in = 50.4% and = 58.2% for the Ku et al. and Boulanger and Idriss relationships, respectively. Similarly, a  value of 0.3 results in = 99.9% and = 99.1% for the Ku et al. and Boulanger and Idriss relationships, respectively. From these  values, the actual liquefaction initiation hazard can more accurately be analyzed, as the = 0.9 prediction indicated that the soil will be about as likely to liquefy as to not liquefy, while the = 0.3 prediction indicates that liquefaction is almost guaranteed. These results represent how the actual liquefaction hazard can vary significantly even though predictions of liquefaction initiation using differentmethods appear to agree.

To see the full effect of the conversion to the realm of, Figure 3-24 and Figure 3-25 show examples of the comparison of  at a return period of 475 years. In the Ku et al. comparison, out of the 45,590 predictions, 42 percent of the performance-based values predicted a lower liquefaction hazard compared to the conventional method, while 26% percent predicted a lower hazard for the conventional method. The remainder of the values computed an equivalent hazard with both methods. For the Boulanger and Idriss methods, the performance-based method predicted a lower liquefaction hazard 23% of the time compared to the conventional method which predicted a lower liquefaction hazard 18% of the time. The remainder of predictions gave equivalent values for the predicted liquefaction hazard regardless of the method used. This paints   significantly different picture than the  comparisons, as a much larger percentage of the analysis could potentially calculate a reduced liquefaction hazard with use of the performance-based method.

PL for Full-Probabilistic Ku et al.

PL for Pseudo-Probabilistic Robertson and Wride (Mean)

Figure 3-24: Probability of liquefaction values from study (using Ku et al. equation)  = 475.

PL for Full-Probabilistic Boulanger and Idriss

PL for Pseudo-Probabilistic Boulanger and Idriss (Mean)

Figure 3-25: Probability of liquefaction values from study (using Boulanger and Idriss equation)  = 475.

Even though only a relatively small percentage of cases disagree in the prediction of liquefaction when considering, by looking at the, which is facilitated by a probabilistic framework, the conventional analysis on average will predict a slightly greater liquefaction hazard. This means that in many cases the conventional analysis would require a larger amount of ground improvement to bring the soil to a satisfactory state to resist liquefaction and related hazards. This also signifies the possibility that the use of a performance-based method could provide cost savings in a potential design.

## Settlement Results

This section presents all results from the study for calculated liquefaction-induced settlement estimates. Discussion of all settlement results is also included.

### Example of Results Calculated by CPTLiquefY

After CPTLiquefY has completed a full-probabilistic analysis, the settlement results can be viewed. Under the “Settlement Results” tab, the data for the total ground surface settlement hazard curve is shown. This hazard curve represents the expected settlement values, at that site, for return periods ranging from the 475 year to the 10,000 year return period. This data is easily copied and pasted into Excel for plotting. An example of a full-probabilistic settlement hazard curve is shown in Figure 3-26.

Figure 3-26: Example settlement hazard curve.

### Comparison of Full-Probabilistic, Pseudo-Probabilistic, and Semi-Probabilistic

First, in areas of low seismicity the pseudo-probabilistic methods calculated about 10-50% higher settlement values than the PBEE method. This relationship is logical because the pseudo-probabilistic method does not take into account the low probability of large earthquakes occurring. However, with higher return periods, the PBEE method predicted about 5-50% more settlement than pseudo-probabilistic procedures. Second, in areas of medium to high seismicity the pseudo-probabilistic method predicts similar settlement values to the full-probabilistic method for lower return periods, but about 30-100% less settlement values at higher return periods. The higher the return period and the seismicity of a city, the more the pseudo-probabilistic method under-predicts settlements. These trends reflect those found in the SPT study. Finally, by comparing the liquefaction triggering methods, both methods generally calculate similar settlement values, but the Boulanger and Idriss method proves to be more conservative than the Robertson and Wride method.

A comparative study is also performed for the semi-probabilistic settlement approach and the PBEE settlement approach. Similar trends observed from the pseudo-probabilistic comparative study are observed from the semi-probabilistic approach. However, at the higher return periods, the semi-probabilistic approach tends to under-predict settlements marginally more than the pseudo-probabilistic approach. Also, across all return periods, the data scatter for the semi-probabilistic approach is tighter than the pseudo-probabilistic results. This trend indicates a slightly higher level of consistency and efficiency as compared to the pseudo-probabilistic approach.

To see these trends visually, comparison plots were created to compare the pseudo-probabilistic and semi-probabilistic results to the full-probabilistic results. These plots are presented in Figure 3-27 through Figure 3-44. Figure 3-27 through Figure 3-35 represent all cities and profiles using the Robertson and Wride (2009) method, with the best fit line shown in blue. Figure 3-36 through Figure 3-44 represent all cities and profiles using the Boulanger and Idriss (2014) method, with the best fit line shown in red. Also included on each comparison plot is a one-to-one line, shown in black. If the pseudo-probabilistic or semi-probabilistic methods correlated perfectly with the full-probabilistic methods the data would fall directly on the one-to-one line. However, a data trend line that falls above the one-to-one line indicates an over-prediction of settlement by the pseudo-probabilistic and semi-probabilistic methods and, conversely, when the data trend line plots below the one-to-one line pseudo-probabilistic and semi-probabilistic methods are under predicting settlement.

Figure 3-27: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 475 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 3-28: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 1039 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 3-29: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 2475 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 3-30: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 475 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 3-31: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 1039 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 3-32: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 2475 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 3-33: Semi-probabilistic versus fully-probabilistic for the 475 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 3-34: Semi-probabilistic versus fully-probabilistic for the 1039 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 3-35: Semi-probabilistic versus fully-probabilistic for the 2475 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 3-36: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 475 year return period using the Boulanger and Idriss liquefaction triggering procedure.

Figure 3-37: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 1039 year return period using the Boulanger and Idriss liquefaction triggering procedure.

Figure 3-38: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 2475 year return period using the Boulanger and Idriss liquefaction triggering procedure.

Figure 3-39: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 475 year return period using the Boulanger and Idriss liquefaction triggering procedure.

Figure 3-40: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 1039 year return period using the Boulanger and Idriss liquefaction triggering procedure.

Figure 3-41: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 2475 year return period using the Boulanger and Idriss liquefaction triggering procedure.

Figure 3-42: Semi-probabilistic versus fully-probabilistic for the 475 year return period using the Boulanger and Idriss liquefaction triggering procedure.

Figure 3-43: Semi-probabilistic versus fully-probabilistic for the 1039 year return period using the Boulanger and Idriss liquefaction triggering procedure.

Figure 3-44: Semi-probabilistic versus fully-probabilistic for the 2475 year return period using the Boulanger and Idriss liquefaction triggering procedure.

The data trend lines tend to correlate with the one-to-one line fairly well up to a certain level of settlement for each return period when comparing the full-probabilistic approach to the pseudo-probabilistic (mean magnitude) approach. The Robertson and Wride data, from the two settlement approaches, tend to match up well until about 20cm, 15cm, and 10cm of settlement for the 475 year, 1039 year, and 2475 year return periods, respectively. While the Boulanger and Idriss data, from the two settlement approaches, tend to line up fairly well until about 10cm, 5cm, and 3cm for the 475 year, 1039 year, and 2475 year return periods, respectively. The modal magnitude pseudo-probabilistic approach follows similar trends, but digress from the one-to-one line at lower values. The Robertson and Wride modal magnitude pseudo-probabilistic data lines up with the PBEE data fairly well until about 13cm, 10cm, and 6cm for the 475 year, 1039 year, and 2475 year return periods, respectively. While the Boulanger and Idriss data averages line up fairly well until about 8cm, 6cm, and 5cm for the 475 year, 1039 year, and 2475 year return periods, respectively.

To further examine the source of the trends mentioned above, the settlement values computed from pseudo and semi-probabilistic methods, for the 1039 year and 2475 year return periods, were entered into the probabilistic hazard curve to back-calculate the actual return period associated with that settlement value. The results of this process are summarized as box and whisker plots in Figure 3-45 and Figure 3-46.

Actual Return Period (years)

Figure 3-45: Box and whisker plots of actual return periods versus assumed 1039 year return period in the settlement analysis.

Actual Return Period (years)

Figure 3-46: Box and whisker plots of actual return periods versus assumed 2475 year return period in the settlement analysis.

The box and whisker plots shown in Figure 3-45 and Figure 3-46 illustrate the median, first and third quartiles, maximum and minimum values, and the average (marked by an “x”) values of the return periods. As noted previously, these values represent actual return periods because they are generated from the full-probabilistic settlement hazard curve. The assumed return period is presented as a red dashed line for reference.

The results for the 1039 return period box plots (Figure 3-45) seem to match the results from Figure 3-27 through Figure 3-44 fairly well. The one-to-one plots and the box and whisker plots both indicated a fairly good match between approaches for the 1039 return period. The results from the 2475 return period (Figure 3-46) also line up with the results from the one-to-one plots, by indicating an under-prediction of post-liquefaction settlements from conventional standards. The data suggest that, on average the deterministic analyses underestimate the seismic hazard by 50% when considering a larger seismic event.

To examine these trends even further, a heat map was generated (Figure 3-47). Each number in the map represents the number of CPT soundings, out of the total 20, in which the pseudo-probabilistic approach predicted less settlement than the full-probabilistic approach. These values are presented for both triggering procedures, each return period, and both magnitudes (mean and modal) at each city. The cities are ordered from the lowest seismicity to the highest from top down.

Figure 3-47: A heat map representing the number of CPT soundings, out of 20 soundings, in which the pseudo-probabilistic method under predicted settlement compared to the full-probabilistic procedure.

The heat map reinforces the trends observed above. The heat map illustrates that the Boulanger and Idriss triggering procedure is under predicting settlement at a higher percentage than the Robertson and Wride method. This trend can also demonstrate how the Boulanger and Idriss method produces larger PBEE settlement values than the Robertson and Wride method. The heat map reinforces the trend indicating that the under prediction of settlement increases with an increasing level of site seismicity and return period. At the 2475 year return period, the Robertson and Wride method is under predicting settlement at about 50% of the soundings. While the Boulanger and Idriss procedure under predicted settlements for almost 90% of the CPT soundings at the 2475 return period.

The heat map points out 3 outliers at Butte, MT for the modal magnitude pseudo-probabilistic values where all 20 profiles under predicted settlement for all three return periods. This discrepancy is likely due to a large difference in the MSF due to a significant difference in the mean and modal magnitudes at Butte, MT (Table 3-2). This discrepancy does not manifest in the Boulanger and Idriss procedure because of the differing MSF calculation methods. The Boulanger and Idriss procedure does depend solely on the magnitude to calculate the MSF like the Robertson and Wride method.

At the 475-year return period pseudo-probabilistic method generally over-predicted or the methods predicted similar settlement values. This trend is logical because the pseudo-probabilistic procedure uses a deterministic method of predicting settlements. Deterministic methods are considered to be a conservative approach because typically consider the controlling scenario earthquake.

However, this idea is concerning after reviewing the results from the higher return periods. The data suggests, as the return period increased, the pseudo-probabilistic analyses under-predicted the level of liquefaction-induced settlement hazard by up to 90%. This trend is likely caused by the fact that the pseudo-probabilistic and semi-probabilistic methods compute post-liquefaction volumetric strains deterministically. Deterministic strain calculations ignore the inherent uncertainty associated with calculating strain values. The full-probabilistic procedure, however, accounts for this uncertainty, resulting in higher settlement estimations.

Results from the semi-probabilistic approach had less scatter but underestimated settlements at about the same percentage as the pseudo-probabilistic approach. The trends depicted in the semi-probabilistic results are very similar to the trends from the pseudo-probabilistic results. These similarities in the two methods’ results suggests the uncertainty in the liquefaction triggering is less significant than the uncertainty in the strain calculations. These results also prove the semi-probabilistic method is not an improvement to the current pseudo-probabilistic methods. Significant calculations are required to perform the calculations necessary for the semi-probabilistic approach and, therefore, may not be worth it for engineers to use this approach as a replacement to the pseudo-probabilistic approach. Engineers should either continue using the easier and equally accurate pseudo-probabilistic approach, or go all the way to the full-probabilistic approach for more accuracy.

By comparing the results a comparison can be made between the Robertson and Wride and Boulanger and Idriss triggering methods. The two methods consistently predicted similar full-probabilistic settlements for the varying CPT soundings and cities. However, the Boulanger and Idriss procedure consistently produced larger settlement values. This trend indicates the Boulanger and Idriss method is a more conservative option than the Robertson and Wride method.

Pseudo-probabilistic methods are widely accepted in industry because they are considered to be a simple way to incorporate probabilistic ground motions into the liquefaction analysis. If, in fact, these methods are under-predicting liquefaction-induced settlements, then relying on pseudo-probabilistic methods for design presents a dangerous risk. This data suggests that pseudo-probabilistic design methods are a conservative option for areas of lower seismicity and lower return periods. However, according to this data, fully-probabilistic methods should be used for regions of higher seismicity and when designing for medium to large seismic events.

## Lateral Spread Results

Presented in this section is the discussion of results of the lateral spread analysis. The general trends remain consistent with those found in the settlement results. However, as a different model is used in calculating lateral spread displacements there are some differences from the trends found in the settlement results. These differences will be emphasized in this section. As mentioned in the methodology section, two lateral spread geometries were analyzed. The first geometry was that of a gently sloping ground with a ground slope of 3% (referred to as lateral spread geometry 1). The second geometry was a level ground with a free face, with the height of the free face, H, set to 6m, and the length to the toe, L, set to 50m (referred to as lateral spread geometry 2).

### Example of Results Calculated by CPTLiquefY

After CPTLiquefY has completed a full-probabilistic analysis, the lateral spread results can also be viewed under the “Lateral Spread Results” tab. Similar to the settlement results, the data for a hazard curve is then displayed, showing estimated lateral spread results for return periods of 475 to 10,000 years. An example hazard curve for lateral spread is shown in Figure 3-48.

Figure 3-48 Example of lateral spread hazard curve.

### Comparison of Full-Probabilistic, Pseudo-Probabilistic, and Semi-Probabilistic

Just as in the settlement analysis, the pseudo probabilistic and full-probabilistic results were compared and trends were identified. The general trends were similar to those found in the settlement results. In areas of higher seismicity (e.g., San Francisco, Santa Monica, Eureka) the full-probabilistic method calculated higher lateral spread displacements than the pseudo-probabilistic method, especially at higher return periods. In areas of lower seismicity, both the pseudo-probabilistic and full-probabilistic calculated similar values at higher return periods, but the full-probabilistic method resulted in lower values at lower return periods. In general, the higher the seismicity of a site and the higher the return period, the more the pseudo-probabilistic method under-predicted the lateral spread, for both mean and modal magnitude calculations.

The two liquefaction triggering methods followed consistent trends to those seen in the settlement results. Lateral spread displacements calculated using the Boulanger and Idriss procedure consistently showed more conservative values (for any method) than the Robertson and Wride procedure.

When comparing the semi-probabilistic and full-probabilistic results for lateral spread, the same trends found in the settlement results remain true. That is, the semi-probabilistic results show similar trends found in the pseudo-probabilistic comparison, but tend to show slightly lower values of lateral spread than the pseudo-probabilistic results. Also, there is much less scatter in this data, indicating more consistent results in the semi-probabilistic method. However, the semi-probabilistic approach proved to have similar levels of accuracy as the pseudo-probabilistic approach. Therefore, the semi-probabilistic approach is not an improvement to the pseudo-probabilistic approach.

Comparison plots for lateral spread are shown in Figure 3-49 through Figure 3-66, showing the relationship between full-probabilistic results and the pseudo-probabilistic or semi-probabilistic results. One thing to note is that the trend line for the Robertson and Wride values in the pseudo-probabilistic comparisons lies slightly above the one-to-one line for the 475-year return period. This indicates that at low return periods, the full-probabilistic method under-predicts lateral spread displacements. While this is different than what was found in the settlement results, it is still consistent with the general trends seen. The “Robertson Data” trend line in this instance for settlement is below the one-to-one line due to a single value (soil profile 1 at San Jose). If this value were removed, the trend line for settlement would look identical to the trend line for lateral spread. That point was not as far from the general trend for lateral spread, and therefore did not have as big of an effect.

The comparison plots in Figure 3-49 through Figure 3-66 are only for lateral spread geometry 1. Lateral spread geometry 2 showed the exact same trends, only the values were scaled down. The comparison plots for lateral spread geometry 2 can be found in the appendix.

Figure 3-49: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 475 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 3-50: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 1039 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 3-51: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 2475 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 3-52: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 475 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 3-53: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 1039 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 3-54: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 2475 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 3-55: Semi-probabilistic versus fully-probabilistic for the 475 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 3-56: Semi-probabilistic versus fully-probabilistic for the 1039 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 3-57: Semi-probabilistic versus fully-probabilistic for the 2475 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 3-58: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 475 year return period using the Boulanger and Idriss liquefaction triggering procedure.

Figure 3-59: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 1039 year return period using the Boulanger and Idriss liquefaction triggering procedure.

Figure 3-60: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 2475 year return period using the Boulanger and Idriss liquefaction triggering procedure.

Figure 3-61: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 475 year return period using the Boulanger and Idriss liquefaction triggering procedure.

Figure 3-62: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 1039 year return period using the Boulanger and Idriss liquefaction triggering procedure.

Figure 3-63: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 2475 year return period using the Boulanger and Idriss liquefaction triggering procedure.

Figure 3-64: Semi-probabilistic versus fully-probabilistic for the 475 year return period using the Boulanger and Idriss liquefaction triggering procedure.

Figure 3-65: Semi-probabilistic versus fully-probabilistic for the 1039 year return period using the Boulanger and Idriss liquefaction triggering procedure.

Figure 3-66: Semi-probabilistic versus fully-probabilistic for the 2475 year return period using the Boulanger and Idriss liquefaction triggering procedure.

Figure 3-49 through Figure 3-66 clearly show the difference in the results when using a full-probabilistic analysis of lateral spread. The results seen in the lateral spread analysis are very similar to those seen in the settlement analysis previously discussed, however while the plots of the results may look similar there are some key differences to note. In general there is more scatter in the data when compared to the results from the settlement analysis. Because of this scatter there seems to be very few times when the data follows the one-to-one line and matches the conventional approach to the full-probabilistic approach. For all return periods and each different conventional approach it can be seen that the Boulanger and Idriss data plots below the Robertson and Wride data. This indicates that the Boulanger and Idriss method produces more values that fall below the one-to-one line, or in other words are being under-predicted by the conventional approach.

When looking at the comparison between the pseudo-probabilistic and full-probabilistic approaches it can be seen that the trend for both methods begins above the one-to-one line, and at times doesn’t even cross below it. The Robertson and Wride approach crosses below the one-to-one line at 200cm, and 100cm for the 1039 year, and 2475 year return periods, respectively (for the 475 year return period it stays consistently above the one-to-one line). The Boulanger and Idriss approach crosses below the one-to-one line at 400cm, 800cm, and 500cm for the 475 year, 1039 year, and 2475 year return periods, respectively. These trends were consistent between mean and modal magnitude pseudo-probabilistic analyses. The semi-probabilistic analysis provided very similar results, with trends beginning above the one-to-one line and eventually going below it.

As with the analysis of settlement, to further examine the trends and results from the lateral spread analysis the values for the 1039 year and 2475 year return periods were entered into the probabilistic hazard curve to back-calculate the actual return period associated with that lateral spread displacement. The results of this process are summarized as box and whisker plots in Figure 3-67 and Figure 3-68.

Actual Return Period (years)

Figure 3-67: Box and whisker plots of actual return periods versus

assumed 1039 year return period in the lateral spread analysis.

Actual Return Period (years)

Figure 3-68: Box and whisker plots of actual return periods versus

assumed 2475 year return period in the lateral spread analysis.

The box and whisker plots shown in Figure 3-67 and Figure 3-68 illustrate the median, first and third quartiles, maximum and minimum values, and the average (marked by an “x”) values of the return periods. As noted previously, these values represent actual return periods because they are generated from the fully-probabilistic settlement hazard curve. The assumed return period is presented as a red dashed line for reference.

In Figure 3-67 it can be seen that the actual return periods are fairly close to the assumed return period of 1039 years. While there is significant scatter in the box plots, the median value of calculated return period falls right along with the assumed return period of 1039 years. This is true for all three different analysis methods. For the 2475 year return period shown in Figure 3-68 it can be seen that the median assumed return period is closer to 1200 years, indicating that there is significant under-predication of lateral spread displacements when using the conventional analysis methods. These results are fairly consistent with what was found in the one-to-one plots, showing that as the return period increases the amount of under-prediction of lateral spread displacements using conventional methods also increases.

Further examination of the trends in the lateral spread analysis can be done by generating a heat map as was created in Figure 3-47 for the settlement analysis. This heat map was created, but not included in this report as the trends observed were identical to that created for settlement. For an explanation of these trends please refer to Section 3.3.2.

In general it was found that the pseudo-probabilistic approach of predicting lateral spread displacements typically over-predicted compared to the full-probabilistic approach for lower return periods (475 years) and lower seismic areas. Conversely, the pseudo-probabilistic approach typically under-predicted compared to the full-probabilistic approach for higher return periods (2475 years) and higher seismic areas. The trends show that Robertson and Wride procedure consistently had lower displacements than the Boulanger and Idriss procedure when looking at the full-probabilistic approach. This indicates that while the Robertson and Wride procedure may be a closer fit to more conventional methods, the Boulanger and Idriss procedure is a more conservative full-probabilistic approach when comparing the two together. The semi-probabilistic approach showed these similar trends as mentioned for the pseudo-probabilistic approach, with less scatter in the data, and is not a significant improvement on the pseudo-probabilistic method.

Based on these results it can be seen that while the pseudo-probabilistic method may be accepted in industry, it has its shortcomings when compared to the full-probabilistic approach. For areas of lower seismicity and lower return periods, the pseudo probabilistic approach may be adequate. But for areas of higher seismicity and higher return periods the use of the pseudo-probabilistic approach poses a risk as the lateral spread displacements could possibly be under-predicted. In these cases it is recommended that the full-probabilistic method be used.

# SENSITIVITY ANALYSIS

In addition to the comparative study, a sensitivity analysis was performed to test the effect of correction factors on the full-probabilistic procedure for settlement and lateral spread. This analysis was performed for the thin layer correction and depth weighting factor correction. These results can show the impact and importance these factors can have on the newly developed full-probabilistic procedure.

## Thin Layer Correction and Depth Weighting Factor

When dealing with all levels of probability, some unrealistic or impossible strain values can be computed. Various correction methods have been developed to address and correct unrealistic strain values.

### Depth Weighting Factor Correction

The likelihood of a soil to liquefy decreases with depth (Iwasaki et al., 1982) . A depth weighting factor (DF) was developed to account for this phenomenon. This depth factor aids in producing a better fit between models and case studies and is based on the following: (1) the triggering of void ratio redistribution, resulting in unfavorably higher void ratios for shallower layers from upward seepage; (2) reduced induced shear stresses and number of shear stress cycles transmitted to deeper soil layers due to initial liquefaction of surficial layers; and (3) possible arching effects due to nonliquefied soil layers (Cetin et al., 2009). For these reasons, it is assumed that the contribution of layers to ground surface settlement and lateral spread diminishes with depth. After statistical assessments, 18m is the critical depth at which the soil layers past this point will not affect the ground surface settlement. This depth factor can be computed as

| DFi=1-di18m | (4-1) |
| --- | --- |

where di is the depth of the specific soil layer.

For settlement and lateral spread displacement of gently sloping ground, DF is then multiplied by the individual strain at each layer. This scales down the strain, resulting in more realistic settlement and lateral spread values. However, for lateral spread of a level ground with free face, a depth factor is not calculated. If the depth of the layer is greater than 2 times the height of the free face, the strain from that layer is considered to not contribute to the lateral spread.

### Thin Layer Correction

Figure 4-1: Tip resistance analysis for thin sand layer (deposit A) interbedded within soft clay layer (deposit B). (Ahmadi & Robertson, 2005).

When a thin sand layer is embedded within a soft clay, the cone from the CPT will read the sand layer’s cone tip resistance as much lower than the actual stiffness of the thin layer because it has started to pick up the soft clay layer’s resistance early (Ahmadi & Robertson, 2005). This discrepancy results in an over-prediction of post-liquefaction settlements and lateral spread displacements because the cone is reading the soil as softer than its actual stiffness. A correction factor is suggested to correct the cone tip resistance in the thin sand layers. As shown in Figure 4-1, as the cone enters deposit A (thin sand layer), the soil resistance is significantly reduced before the cone reaches deposit B (soft clay layer). This phenomena occurs because the cone is picking up the softness of deposit B before it reaches deposit B. The higher the stiffness and thinner the layer of sand interbedded within soft clay, the larger the thin layer correction factor will be.

Once a layer has been identified as needing the thin layer correction, the tip resistance can be adjusted with a correction factor. A layer is identified as a thin layer if there is a steep change in the soil behavior index (Ic), usually a change of 0.01 or greater, for multiple soil sublayer increments, usually 4 (Robertson, 2011). Once identified, these layer’s tip resistance can be corrected using

| qc*=KHqc | (4-2) |
| --- | --- |

where qc* is the corrected cone tip resistance and KH is the correction factor (Youd, 2001). This factor can be calculated by

| KH=0.25Hdc17-1.772+1.0 | (4-3) |
| --- | --- |

where dc is the diameter of the cone, and H is the layer thickness.

## Sensitivity Analysis of Settlement

Four separate full-probabilistic settlement calculations were performed, for each city and CPT profile, to test the sensitivity of the full-probabilistic procedure to different correction factors. These four calculations included a baseline (i.e., no corrections applied), a run with the depth weighting factor, a run with the thin layer correction, and a run with both the thin layer and depth factor correction applied. All 20 soil profiles, at all of the cities, and for all three return periods were ran for each of these options. The data was combined and is presented in box and whisker plots below (Figure 4-2 through Figure 4-4)

To create these box and whisker plots, a ratio (R) was created, which represents the ratio of each of the separate options calculated settlement values to the baseline corrected settlement values. This means that if R is equal to 1, the specific correction factor had no impact on the settlement calculation. Therefore, the further away R is from 1, the more sensitive the probabilistic procedure is to that particular option.

The box and whisker plots show the impact each correction factor has on the full-probabilistic procedure. The box and whisker plots prove the importance and impact of correction factors. Based on the plots, it is apparent the full-probabilistic procedure is more sensitive to the depth weighting factor than the thin layer correction factor. However, the depth factor correction experienced a larger spread than the thin layer correction. It is logical that the thin layer correction had a smaller impact on the calculated settlement values. For the thin layer correction to even be applied the soil profile needed to contain thin sand layers interbedded within soft clay layers. A few of the 20 profiles did not have this criteria and therefore did not experience any thin layer correction. However, because the depth weighting factor is independent of soil type, the depth weighting factor was always applied.

The data suggests that return period does not affect the sensitivity significantly. Across all three return periods, the median value and general trends are about the same. This is logical because the degree of sensitivity is dependent on soil type for thin layer correction and the depth factor correction will be constant because it is only dependent on depth. All of which were constant for each settlement calculation. However, as the return period increased, the scatter decreased.

Figure 4-2: Box and whisker plots for R (settlement) at a return period of 475 years.

Figure 4-3: Box and whisker plots for R (settlement) at a return period of 1039 years.

Figure 4-4: Box and whisker plots for R (settlement) at a return period of 2475 years.

## Sensitivity Analysis of Lateral Spread

As in the sensitivity analysis of settlement, the sensitivity analysis of lateral spread was performed comparing the results of the baseline probabilistic method (i.e. no corrections applied), depth weighting factor, thin layer correction, and both thin layer and depth factor correction. This was also done for all 20 CPT profiles, 10 cities and 3 return period. Box and whisker plots were created to illustrate this comparison. The box and whisker plots for lateral spread geometry 1 are displayed in Figure 4-5 through Figure 4-7. R is the ratio of the indicated correction option of lateral spread to the baseline lateral spread.

Figure 4-5: Box and whisker plots for R (lateral spread geometry 1) at a return period of 475 years.

Figure 4-6: Box and whisker plots for R (lateral spread geometry 1) at a return period of 1039 years.

Figure 4-7: Box and whisker plots for R (lateral spread geometry 1) at a return period of 2475 years.

From the box and whisker plots in Figure 4-5 through Figure 4-7 it can be seen that the depth factor has more of an impact on the lateral spread displacements than the thin layer correction. For both procedures and all three return periods, the median R value for depth factor is approximately 0.55, while the median R value for thin layer correction is approximately 0.93 to 0.95. As the return period increased, the main change seen is that the spread of the R values decreases slightly. This is consistent with the results of the settlement sensitivity analysis.

It was desired that the same analysis be done for lateral spread geometry 2. Since geometry 2 is for level ground with a free face and not a gently sloping ground the depth correction is calculated differently. For level ground with a free face (geometry 2), a depth weighting factor is not calculated, simply if the depth of the layer in question is at a depth more than 2 times the height of the free face, H, it does not contribute to lateral spread. The height of the free face for geometry 2 was 6m. The maximum depth of all CPT profiles was 12m. Since none of the CPT profiles had layers at a depth beyond 2H, there was no depth factor correction for lateral spread geometry 2. Because of this it was decided to do the sensitivity analysis with a third geometry (referred to as lateral spread geometry 3).

Lateral spread geometry 3 has a free face height, H, of 3m, and a distance from the toe, L, or 25m. These values were chosen because the value of H needed to be smaller, and also the L/H ratio for lateral spread geometry 3 is the same as that of lateral spread geometry 2. With the same L/H ratio, the same result of lateral spread displacement is calculated. The only difference being that the depth correction calculated as 2H for geometry 3 is not equal to or greater than the depth of the CPT profiles. The box and whisker plots for lateral spread geometry 3 are displayed in Figure 4-8 through Figure 4-10. R is again the ratio of the indicated correction option of lateral spread to the baseline lateral spread.

Figure 4-8: Box and whisker plots for R (lateral spread geometry 3) at a return period of 475 years.

Figure 4-9: Box and whisker plots for R (lateral spread geometry 3) at a return period of 1039 years.

Figure 4-10: Box and whisker plots for R (lateral spread geometry 3) at a return period of 2475 years.

Just as with lateral spread geometry 1, in Figure 4-8 through Figure 4-10 it can be seen that the depth factor has more of an impact on the lateral spread displacements than the thin layer correction. However, the actual impact of the depth factor strongly depends on the number of susceptible and liquefied layers located at deeper depths. For both procedures and all three return periods, the median R value for depth factor is approximately 0.55 to 0.60, while the median R value for thin layer correction is approximately 0.95. As the return period increased, the main change seen is that the spread of the R values decreases slightly. This is consistent with the results of the settlement sensitivity analysis. The spread of the box and whisker plots shown in Figures 4-8 through Figure 4-10 are indicative of the relative uncertainty and sensitivity of the correction factors to differing soil profiles.

# CONCLUSIONS

## Summary

This quarter involved the completion of CPTLiquefY and performing a study comparing the newly developed full-probabilistic procedure for the CPT and conventional hazard analysis methods. CPTLiquefY was almost complete by the end of Quarter 2. This quarter the lateral spread analysis capabilities were completed, as well as some minor corrections.

The comparative study was performed by comparing calculated liquefaction triggering, settlement, and lateral spread estimations for the pseudo-probabilistic, semi-probabilistic, and full-probabilistic hazard analysis methods. Estimated hazard values were calculated for 20 CPT profiles at 10 different cities of varying seismicity. The analysis of liquefaction triggering methods appears to show that in general the Boulanger and Idriss full-probabilistic procedure will give more conservative results than the Ku et al. full-probabilistic procedure. In comparing these new methods with more common forms of liquefaction triggering analysis, it was observed that the full-probabilistic methods on average appear to predict a lower liquefaction hazard when compared to the conventional methods. The lateral spread and settlement analyses produced similar trends. It was observed that for a low return period, the pseudo-probabilistic settlement and lateral spread values correlated fairly well with full-probabilistic settlement values. However, at higher return periods, this correlation deteriorated and showed pseudo-probabilistic methods under-predicting settlements and lateral spreading significantly. The semi-probabilistic method followed similar trends to the pseudo-probabilistic method. These results suggest current design practices unintentionally ignore considerable amounts of risk when estimating liquefaction-induced settlements for earthquakes with higher return periods.

In addition to the comparative study, a sensitivity analysis was performed to test the effect of correction factors on the full-probabilistic estimation procedure for settlement and lateral spreading calculations. This analysis was performed for the thin layer correction and depth weighting factor correction. The results proved the full-probabilistic procedure is more sensitive to the depth weighting factor than the thin layer correction.

## Future Work

This report completes phase one of the project contract. With the completed analysis tool, CPTLiquefY, the second phase of this research can begin. CPTLiquefY will now be used to create simplified procedures, as well as generate probabilistic seismic hazard maps for liquefaction triggering, post-liquefaction settlement, and lateral spreading. The next report will provide more details on this work.

# REFERENCES

Ahmadi, M. M., & Robertson, P. K. (2005). Thin-layer effects on the CPT qc measurement. Canadian Geotechnical Journal, 42(5), 1302-1317.

Boulanger, R., & Idriss, I. (2014). CPT and SPT based liquefaction triggering procedures. Center for Geotechnical Modeling, Department of Civil and Environmental Engineering, University of California, Davis, CA. Report No. UCD/CGM-14/01.

Cetin, K., Bilge, H., Wu, J., Kammerer, A., & Seed, R. (2009). Probabilistic Model for the Assessment of Cyclically Induced Reconsolidation (Volumetric) Settlements. Journal of Geotechnical and Geoenvironmental Engineering, 3(135), 387-393.

Franke, K. W., Lingwall, B. N., & Youd, T. L. (2014). Sensitivity of empirical liquefaction assessment to seismic loading in areas of low seismicity and its implications for sustainability. Paper presented at the Geo-Congress 2014 Technical Papers: Geo-characterization and Modeling for Sustainability.

Franke, K. W., Wright, A. D., & Hatch, C. K. (2014). PBLiquefY: A New Analysis tool for the performance-based evaluation of liquefaction triggering. Paper presented at the 10th National Conference on Earthquake Engineering.

Iwasaki, T., Tokida, K., Tatsuka, F., Wantabe, S., Yasuda, S., & Sato, H. (1982). Miccrozonation for soil liquefaction potential using the simplified methods. Paper presented at the 3rd International Conference on Microzonation, Seattle.

Kramer, S. L., & Mayfield, R. T. (2007). Return period of soil liquefaction. Journal of Geotechnical and Geoenvironmental Engineering, 133(7), 802-813.

Ku, C.-S., Juang, C. H., Chang, C.-W., and Ching, J. (2012). "Probabilistic version of the Robertson and Wride method for liquefaction evaluation: development and application."

Canadian Geotechnical Journal, 49(1), 27-44.

Robertson, P. (2009). Performance based earthquake design using the CPT. Paper presented at the Proceedings of IS-Tokyo 2009: international conference on performance-based design in earthquake geotechnical engineering—from case history to practice, Tokyo, Japan.

Robertson, P. (2011). Computing in Geotechnical Engineering-Automatic Software Detection of CPT Transition Zones. Geotechnical News, 29(2), 33.

Robertson, P., & Wride, C. (1998). Evaluating cyclic liquefaction potential using the cone penetration test. Canadian Geotechnical Journal, 35(3), 442-459.

U.S.G.S. Earthquake Hazards Program. Table of CPT Data, All Regions. Retrieved from https://earthquake.usgs.gov/research/cpt/data/table/

Youd, L. (2001). Liquefaction Resistance of Soils: Summary Report from the 1996 NCEER and 1998/NSF Workshops on Evaluation of Liquefaction Resistance of Soils. Journal of Geotechnical and Geoenvironmental Engineering, 127(4).

Zhang, G., Robertson, P. K., & Brachman, R. (2004). Estimating liquefaction-induced lateral displacements using the standard penetration test or cone penetration test. Journal of Geotechnical and Geoenvironmental Engineering, 130(8), 861-871.

# APPENDIX

Figure 6-1 through Figure 6-18 show the comparison of the full-probabilistic method to the pseudo probabilistic method or semi-probabilistic method for lateral spread geometry 2.

Figure 6-1: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 475 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 6-2: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 1039 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 6-3: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 2475 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 6-4: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 475 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 6-5: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 1039 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 6-6: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 2475 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 6-7: Semi-probabilistic versus fully-probabilistic for the 475 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 6-8: Semi-probabilistic versus fully-probabilistic for the 1039 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 6-9: Semi-probabilistic versus fully-probabilistic for the 2475 year return period using the Robertson and Wride liquefaction triggering procedure.

Figure 6-10: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 475 year return period using the Boulanger and Idriss liquefaction triggering procedure.

Figure 6-11: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 1039 year return period using the Boulanger and Idriss liquefaction triggering procedure.

Figure 6-12: Mean magnitude pseudo-probabilistic versus fully-probabilistic for the 2475 year return period using the Boulanger and Idriss liquefaction triggering procedure.

Figure 6-13: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 475 year return period using the Boulanger and Idriss liquefaction triggering procedure.

Figure 6-14: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 1039 year return period using the Boulanger and Idriss liquefaction triggering procedure.

Figure 6-15: Modal magnitude pseudo-probabilistic versus fully-probabilistic for the 2475 year return period using the Boulanger and Idriss liquefaction triggering procedure.

Figure 6-16: Semi-probabilistic versus fully-probabilistic for the 475 year return period using the Boulanger and Idriss liquefaction triggering procedure.

Figure 6-17: Semi-probabilistic versus fully-probabilistic for the 1039 year return period using the Boulanger and Idriss liquefaction triggering procedure.

Figure 6-18: Semi-probabilistic versus fully-probabilistic for the 2475 year return period using the Boulanger and Idriss liquefaction triggering procedure.